YO Log PRO v17.1

Full Edition

User Manual — English Edition

Professional Multi-Contest Amateur Radio Logger

Dezvoltat de: Ardei Constantin-Cătălin (YO8ACR)

Email: yo8acr@gmail.com

Repo: github.com/acc1311/YOLogPRO

Versiune: v17.1 — Full Edition (2025)

Python 3.6+ / Tkinter GUI

Windows 7/8/10/11 | Linux | macOS

# 1. Introduction

YO Log PRO is a professional logging application for amateur radio operators, designed specifically for national and international contests. The program runs on Windows, Linux, and macOS and is written in Python 3 with a Tkinter graphical interface.

The application enables complete logging of radio contacts (QSOs), automatic score calculation, export to standard formats (ADIF, Cabrillo, CSV), CAT transceiver control, and many other advanced features.

## 1.1 System Requirements

| Component | Minimum | Recommended |
| --- | --- | --- |
| Python | 3.6+ | 3.10+ |
| Operating System | Windows 7 / Linux / macOS | Windows 10/11 |
| RAM | 256 MB | 512 MB+ |
| Disk Space | 50 MB | 100 MB+ |
| Screen Resolution | 1024×768 | 1280×800+ |
| DPI Scaling | 100% — 200% | 100% sau 125% |

## 1.2 Installation and Startup

- Install Python 3.6 or newer from python.org

- Download the file yo_log_pro_v171.py

- Run from terminal: python yo_log_pro_v171.py

- On first launch, fill in your station details (callsign, locator, county)

💡 The pyserial module is optional and only required for CAT control via serial port.

# 2. Main Interface

The main window is divided into several functional areas, each with a well-defined role.

## 2.1 Status Bar (top)

The top bar displays in real time:

- Online/Offline UTC — internet connection status and current UTC time

- Active station callsign (e.g. YO8RJU)

- Active log type (Simple Log / Contest)

- Operator category (Individual / Multi-op)

- Total number of logged QSOs

- CAT status (CAT: ON / CAT: OFF)

## 2.2 QSO Entry Area

This is the main working area where each contact's data is entered:

| Field | Description | Shortcut |
| --- | --- | --- |
| Callsign | Callsignul stației contactate | Auto-focus after LOG |
| Freq (kHz) | Frequency in kilohertz | Auto-filled from CAT |
| Bandă | Radio band (e.g. 160m, 80m, 40m...) | F2 — change band |
| Mod | Emission mode (SSB, CW, FT8...) | F3 — change mode |
| RST S | Sent report (e.g. 59) | — |
| RST R | Received report (e.g. 59) | — |
| Notă/Locator | Free note or station locator | — |
| Dată/Oră | Date and UTC time (auto) | Manual if checked |
| Categorie | Contest category | — |

💡 Band and mode do NOT reset between QSOs. They stay at previous values until manually changed.

## 2.3 Main Buttons

| Buton | Function | Shortcut |
| --- | --- | --- |
| LOG | Saves the current QSO to log | Enter |
| Reset | Clears the entry fields | — |
| Callbook | Looks up information about the callsign | — |

## 2.4 QSO Table

The central table displays all logged QSOs with:

- Nr. — sequence number (descending, most recent on top)

- Callsign, Frecvență, Bandă, Mod, RST S, RST R

- Note, Country (auto DXCC), Date, Time, QSO Points

- Special colors: red = duplicate QSO, yellow = new multiplier

## 2.5 Filter Bar

Above the table there are quick filters:

- Band: filter QSOs by band

- Mode: filter by emission mode

💡 Filters affect display only, they do not delete QSOs from the log.

# 3. Button Bar (Toolbar)

The button bar at the bottom of the main window contains two rows with all application functions. On small screens, the bar becomes horizontally scrollable.

## Row 1 — Main Functions

| Buton | Function |
| --- | --- |
| ⚙ Setări | Station config, font, sounds, scroll |
| 🏆 Concursuri | Contest manager — create, edit, select |
| 📡 CAT | CAT transceiver configuration and control |
| 📝 Log Nou | Create a new log for another contest |
| 🎨 Teme | Change the interface color theme |
| 📊 Statistici | Shows detailed statistics of the current log |
| ✔ Validează | Validates the current log against contest rules |
| 📤 Export | Export the log to ADIF, Cabrillo or CSV |

## Row 2 — Advanced Functions

| Buton | Function |
| --- | --- |
| 📥 Import | Import log from ADIF, CSV or Cabrillo |
| ↩ Undo | Undo the last added QSO |
| 💾 Backup | Manually create a log backup |
| 🔍 Caută | Search callsigns in log |
| ⏱ Timer | Contest timer with audio alerts |
| 📝 Log Editor | Dedicated editor with full QSO editing |
| 🌐 Callbook | Search radioamator.ro and QRZ.com |

💡 On screens under 1200px wide, the button bar switches to compact mode with smaller font. Horizontal scroll available with Shift+MouseWheel.

# 4. Settings

The settings window opens from the ⚙ Settings button and contains the complete station configuration.

## 4.1 Station Data

| Field | Description | Exemplu |
| --- | --- | --- |
| Callsign | Callsignul propriu al stației | YO8RJU |
| Locator | Maidenhead locator (6 characters) | KN37GS |
| Județ | County code for YO contests | NT |
| Adresă | Station mailing address | Piatra Neamț, NT |
| Operator | Operator name | Ion Popescu |
| Putere (W) | Transmission power in watts | 100 |
| Email | Email address for exports | yo8rju@example.com |
| Soapbox | Free comment for Cabrillo export | 73 GL |
| Font Size | Interface font size (9-14) | 11 |

## 4.2 Additional Options

- Enable sounds — enable/disable audio alerts

- Enable scroll on popup windows — adds scrollbars on both axes to all windows

💡 The scroll option is useful on small screens or at 100% DPI scaling where some windows may be clipped.

# 5. Contests and Logs

## 5.1 Predefined Contests

The application comes with the following predefined contests:

| ID | Name | Scoring Mode |
| --- | --- | --- |
| simplu | Simple Log (no rules) | No scoring |
| maraton | Ion Creangă Marathon | Marathon (special) |
| stafeta | Amateur Radio Relay | Per QSO |
| yodxhf | YO DX HF Contest | Per band + multipliers |
| yovhf | YO VHF Contest | Distance (km) |
| fieldday | Field Day | Per QSO |
| sprint | Sprint Contest | Per QSO |

## 5.2 Contest Manager

From the 🏆 Contests button you can:

- Select an active contest from the existing list

- Create a new contest with custom rules

- Edit an existing contest

- Delete unnecessary contests

## 5.3 Contest Editor

When creating or editing a contest, you can configure:

| Parameter | Description |
| --- | --- |
| Contest Name | Namea afișată în interfață |
| Cabrillo Name | Namea folosită la export Cabrillo |
| Categories | Categoriesle de operatori acceptate |
| Scoring Mode | Score calculation method |
| Points/QSO | Points awarded per contact |
| Allowed Bands | Radio bands allowed in the contest |
| Allowed Modes | Allowed emission modes |
| Exchange Format | Report exchange format |
| Duration (hours) | Contest duration in hours |

## 5.4 Available Scoring Modes

| Mod | Description |
| --- | --- |
| none | No scoring — log simplu |
| per_qso | Fixed points per each QSO |
| per_band | Different points per band |
| maraton | Special scoring for YO marathons |
| multiplier | QSO × multipliers |
| distance | Points calculated from km distance (locator) |
| custom | Custom rules per callsign |

# 6. Export and Import

## 6.1 Export

From the 📤 Export button you can export the log in three formats:

### ADIF (.adi)

Standard international format for log exchange. Compatible with any logging software (WSJT-X, HRD, DX4WIN, etc.).

### Cabrillo (.log)

Standard format for submitting contest logs to organizers. A configuration dialog appears before export:

- Sent exchange: county, locator, serial number, or none

- Received exchange: from log or none

- Preview before saving — you can verify the content

### CSV (.csv)

Tabular format, compatible with Excel, LibreOffice Calc or any table editor.

⚠️ Before Cabrillo export, validation is performed automatically. Errors are displayed but the export continues.

## 6.2 Import

From the 📥 Import button you can import contacts from:

- ADIF (.adi, .adif) — imports all standard fields

- CSV (.csv) — importă coloanele Callsign, Bandă, Mod, RST, Notă, Data, Ora

- Cabrillo 2.0 and 3.0 (.log) — imports from existing contest logs

⚠️ Import adds QSOs to the current log. Duplicates are not removed automatically.

# 7. CAT Control (Computer Aided Transceiver)

The CAT function enables bidirectional communication between the application and the transceiver: the application automatically reads frequency and mode, and can set the radio frequency.

## 7.1 Supported Protocols

| Protocol | Compatible Transceivers |
| --- | --- |
| Yaesu CAT | FT-817, FT-818, FT-857, FT-891, FT-897, FT-991, FTDX series |
| Icom CI-V | IC-7300, IC-7100, IC-705, IC-7610, IC-9700 și altele Icom |
| Kenwood CAT | TS-480, TS-590, TS-2000, TS-890 |
| Elecraft CAT | KX2, KX3, K3, K3S |
| Hamlib/rigctld | Orice transceiver suportat de Hamlib |
| Manual (fără CAT) | Fără conexiune, frecvența se introduce manual |

## 7.2 CAT Configuration

- Open 📡 CAT from the button bar

- Select the protocol matching your transceiver

- Select the COM port (e.g. COM3 on Windows, /dev/ttyUSB0 on Linux)

- Set the Baud rate (default 38400 for Yaesu, 19200 for Icom)

- Press Connect

- Check the green LED in the status bar — CAT: ON

💡 For Icom CI-V, the default hex address is 94 (IC-7300). Check your transceiver's menu for the correct address.

💡 For Hamlib, start rigctld before connecting: rigctld -m 122 -r COM3 -s 38400

## 7.3 Common CI-V Addresses

| Model Icom | CI-V Address (hex) |
| --- | --- |
| IC-7300 | 94 |
| IC-705 | A2 |
| IC-7100 | 88 |
| IC-7610 | 98 |
| IC-9700 | A2 |

# 8. Contest Timer

The contest timer opens from the ⏱ Timer button and helps track time remaining in the contest.

## 8.1 Usage

- Hours / Min — enter the contest duration (e.g. 4 hours, 0 minutes)

- ▶ Start — starts the timer

- ⏸ Stop — pauses temporarily (resumes from where it stopped)

- ⏹ Reset — resets to zero

## 8.2 Audio Alerts

If the Audio Alerts option is enabled, you will hear:

| Time Remaining | Audio Signal |
| --- | --- |
| 5 minutes | 1 short beep |
| 3 minutes | 2 beeps |
| 1 minute | 3 high beeps |
| Time expired | 5 long beeps |

💡 The timer has scroll on both axes and can be resized.

# 9. Callbook

The Callbook function allows searching for information about any radio callsign from online databases.

## 9.1 Data Sources

- radioamator.ro — Romanian amateur radio database

- QRZ.com — international database (free account required)

## 9.2 Usage

- Introduceți indicativul în câmpul Callsign din fereastra principală

- Press the Callbook button or open from the button bar

- Data is displayed automatically: name, location, country, DXCC

- You can copy the data into the current QSO fields

💡 Callbook requires an internet connection. Without internet, the feature is unavailable.

# 10. Band Map

Band Map displays radio activity in real time, sourced from DX Cluster or your own log.

- Active bands are displayed as columns

- Each spot shows: callsign, frequency, DXCC country

- Click on a spot → frequency is auto-filled in the entry field

- Colors: green = new DXCC, blue = worked, grey = duplicate

💡 Band Map updates automatically when you receive spots from DX Cluster.

# 11. DX Cluster

DX Cluster allows connecting to a telnet DX spot server to see real-time activity on radio bands.

## 11.1 Configuration

- Open DX Cluster from the menu

- Enter the server address (e.g. dxfun.com:8000)

- Enter your own callsign for authentication

- Press Connect

## 11.2 Features

- Display real-time spots in the main table

- Filter by band or DXCC

- Click on spot → auto-fills frequency and callsign in log

- Raw text window displays all raw data from the cluster

# 12. Log Editor

The Log Editor is a dedicated window for complete QSO editing, with more features than the main window.

## 12.1 Available Features

- Complete view of all QSOs

- Direct editing of any field (double-click on cell)

- Delete selected QSOs

- Filter by band, mode, callsign

- Quick search

- Undo for edit operations

- Callbook lookup directly from the editor

- Auto-save after every modification

💡 The Log Editor has scrollbars on both axes and can be freely resized.

# 13. Statistics

The application provides two types of statistics accessible from the button bar.

## 13.1 General Statistics (📊 Statistics)

- Total QSOs and unique callsigns

- Session duration and average QSO/hour rate

- QSOs and points per band

- Total score: QSO Points × Multipliers

## 13.2 QSO Rate Statistics

- QSO/hour chart for each contest hour

- Top DXCC table — most worked countries (with scrollbar)

- Per Band table — QSO distribution across bands (with scrollbar)

- Live stats: total, unique, DXCC, last-hour rate, last-10-minute rate

- Auto-update every 30 seconds

# 14. Quick Reference — Keys and Shortcuts

| Key / Command | Function |
| --- | --- |
| Enter | Save the current QSO (LOG) |
| F2 | Change band (cycles through available bands) |
| F3 | Change emission mode |
| Ctrl+Z | Undo — cancels the last QSO |
| Ctrl+S | Manually save the log |
| MouseWheel | Vertical scroll in windows |
| Shift+MouseWheel | Horizontal scroll in windows |
| Ctrl+F | Search in log |
| Tab | Jump to next field |

# 15. Backup and Saved Data

## 15.1 Auto-Save

The log is saved automatically after each QSO added or modified. Manual saving is not required.

## 15.2 File Locations

| File | Content | Location |
| --- | --- | --- |
| config.json | Station and application settings | Application folder or AppData |
| contests.json | Contest configuration | Application folder |
| log_{id}.json | A specific contest log | Application folder |
| backups/ | Automatic and manual backups | backups/ subfolder |

## 15.3 Manual Backup

- Press the 💾 Backup button in the button bar

- A file backup_YYYYMMDD_HHMMSS_id.json is automatically created in the backups/ folder

- Backup is also created automatically when clearing the log or importing

⚠️ The log clearing operation (🗑 Clear log) is irreversible. Auto-backup is created beforehand, but verify it before confirming.

# 16. Color Themes

The application supports multiple color themes, accessible from the 🎨 Themes button.

| Temă | Description |
| --- | --- |
| Dark (default) | Dark background, light text — ideal for night operation |
| Light | Light background, dark text — ideal for daytime |
| Solarized Dark | Popular theme with warm colors |
| High Contrast | Maximum contrast for enhanced visibility |
| Custom | Main colors can be customized manually |

💡 Theme changes apply immediately without restarting the application.

# 17. Scroll Functionality (UI Patch v17.1)

This feature was added as a UI patch to v17.1 to resolve the issue of clipped windows at 100% DPI scaling or on small-resolution screens.

## 17.1 Enable / Disable

- Open ⚙ Settings

- Check or uncheck the Enable scroll on popup windows option

- Apăsați Salvează

- New windows will open with or without scroll according to the selection

## 17.2 How to Use the Scroll

- Vertical scroll: rotate the mouse wheel (MouseWheel)

- Horizontal scroll: Shift + MouseWheel

- On Linux: Button-4 (up) and Button-5 (down) for vertical scroll

- Scrollbars appear automatically only when content does not fit

## 17.3 Windows with Scroll Enabled

- Timer Concurs

- Setări

- Statistici (Tabelele Top DXCC și Per Bandă)

- Main button bar (horizontal scroll on small screens)

💡 Other windows (Log Editor, Callbook, CAT, etc.) adapt responsively to screen resolution without needing additional scroll.

# 18. Troubleshooting and Common Issues

| Problem | Solution |
| --- | --- |
| Window is clipped on screen | Enable scroll in Settings or increase resolution/reduce DPI scaling |
| CAT does not connect | Check the COM port, Baud rate, and that it is not used by another program |
| Button bar not fully visible | Resize the window or use horizontal scroll (Shift+MouseWheel) |
| Callbook does not work | Check your internet connection |
| Incorrect scores | Check the active contest rules in Contest Manager |
| Log does not save | Check write permissions in the application folder |
| Sounds do not work | Check that the option is enabled in Settings and the system has audio |
| DX Cluster disconnected | Check the server address and internet connection |

73 de YO8ACR — Good luck in the contests!

yo8acr@gmail.com | github.com/acc1311/YOLogPRO